SHARP MZ-1500 Unicard demo disk quick guide

The sample disk contains a sample configuration and several SHARP MZ programs to get you acquainted with the Unicard. The basic program to control the Unicard is called Unicard Manager. It starts automatically. You can use it to directly run MZF/MZT files (BASIC MZF/MZT files must be run from BASIC), map floppy drives, Quick Disks, RAM disks, etc. There are several ways to start the Unicard Manager:

- If /unicard/ramdisc.cfg file exists and points to /MZR/ramdisc.mzr in which the loader is.

- If the contents of the RAM disk are unbootable (e.g. /MZR/rambas.mzr), Unicard will replace it with the loader after MZ reset for a while, until the MZ gets it and then returns the desired contents. The SHARP MZ will thus boot automatically from the spoofed loader. It is also possible to jump to the monitor via RESET+M and start the loader via the EB command. (This is not possible if the RAM disk is marked as bootable but has a CHECK SUM ERROR, e.g. after running BASIC with the bootable RAM disk mapped. In this case, you must reload the corresponding MZR file from the original archive.)

- If there is a /unicard/qd.cfg file pointing to /MZQ/mzfloader.mzq, which contains the loader.

- If the previous file exists but the RAM disk is mapped with faulty contents, it is possible to force the Quick Disk to start via RESET+Q or RESET+M and entering the QL command followed by a double CR.

Sharp MZ-1500 is a little bit specific when comes to Quick Disk (QD:) drive emulation. As MZ-1500 contains physical QD: drive that is on port 0xF4, it cannot be emulated on the same port. You can modify the MZ-1500 hardware and disable the internal QD: drive. In that case we can supple firmware with emulated QD drive on port 0xF4. The easier is to replace the monitor EPROM in MZ-1500 for the one in /EPROM/9Z-502M_eprom74.bin file. This is update monitor, that uses emulated Quick Disk drive on port 0x74. After you replace the ROM, you can easily RUN the QD: files from their MZQ images, even the multidisc ones (dual side games are quite common). When the image file name ends with _A, Unicard replaces it with _B, _C, … automatically, after the QD: side is read. Just wait few seconds and continue, as you would have replace the QD.

SD card contents

/ (root)

unicard.app

- The latest version of the Unicard Mk3b firmware for SHARP MZ-1500.

- Ensures automatic reprogramming of the Unicard firmware on the first start-up.

- You can delete it from the SD card after programming or leave it as it is, it doesn't matter, the Unicard firmware is reprogrammed only once.

Unicard_MZ-1500_SD-sample.doc

- This quick guide. You can remove it from the SD card, Unicard does not use it ;-)

/unicard 		; Unicard configuration folder

emm.cfg

- Contains the path to the mapped EMM: device (EMM RAM disk, default is /MZR/EMM1.mze)

fd0.cfg, fd1.cfg, fd2.cfg a fd3.cfg

- Contains the path to the mapped floppy disk drive A:, B:, C: a D:

mgr1500.mzf

- Unicard Manager – the program that allows you to directly run MZF/MZT files and to configure emulated peripherals mapping.

mzfloader.cfg

- Points to the Unicard Manager MZF executable („/unicard/mgr1500.mzf“ by default).

- It is required for integrated spoofed SRAM disk or loaders in QD: mzfloader.mzq or RAM: ramdisc.mzr, as the loader need to know where wo get the Unicard Manager to run.

network.cfg

- Allows you to set the MAC address, IP address, mask and gateway for an integrated Web or FTP server
default IP: 192.168.1.111
default MASK: 255.255.255.0
default GW: 192.168.1.1

- Also includes a username and password to log in to the integrated FTP server.
default user: unicard
default pass: unic1234

qd.cfg

- Points to the currently mapped QD image: (default loader in MZQ/mzfloader.mzq)

- If the file does not exist, the QD: device is not emulated.

ramdisc.cfg

- Points to the currently mapped RAM: disk (default loader in MZR/ramdisc.mzr).

mz1r23.rom
mz1r24.rom

- Kanji and dictionary ROM contents of SHARP MZ-1R23 and MZ-1R24 interfaces.

/unicard/_not_used/ushelldisabled.cfg

- Disables UniUSARTShell on the USB port and makes the USB available for serial communication on the emulated serial card (RS2:).

- If you move the file one directory up to the /unicard folder, the UniUSARTShell will not be accessible on the USB port for commands and file transfer, but it will be accessible on RS2: channel of the emulated MZ-1E24 serial card. The RS1: channel is always mapped to the physical serial port (DSUB-9 connector on the Unicard back panel).

ATTENTION! If you put files ramdisc.dat or mzfloader.mzq to the /unicard folder, they will be "hard" mapped to RAM: or QD: regardless of any other configuration files and settings in Unicard Manager.

/MZF (programs for easy launching directly from Unicard Manager or loading in Unicard BASIC)

AUTO RUN.mzf

- AUTO RUN file for BASIC_5Z-008_for_unicard.mzf

- Automatically starts when the BASIC starts and makes DIR "SD:"

- Cannot be run from the Unicard Manager, it is a BASIC program

5Z001.MZT

- Common SHARP MZ-1500 BASIC 5Z-001 with support for QD: and RAM: disk.

- We recommend to insert /MZR/rambas.mzr to RAM: device before the launch of the BASIC.

5Z001_QD74.mzf

- Common SHARP MZ-1500 BASIC 5Z-001 with support for emulated QD: on port 0x74 and RAM: disk.

- We recommend to insert /MZQ/qdtestbas.mzq file to QD: and /MZR/rambas.mzr to RAM: device before the launch of the BASIC.

BASIC_5Z-008_for_unicard.mzf

- European SHARP MZ-700 BASIC with Unicard support (“SD:” device), support for “QD:” device was removed.

- As it is European version of BASIC, small Latin characters are screwed up on standard Japanese MZ-1500, as they are on differen positions in ASCII table on Japan and Latin computers. If someone has source codes for 5Z001 BASIC or MZ-1500 Hu-BASIC, we will happily adapt the BASIC for Unicard.

- Extended command are INIT, DIR, SAVE, LOAD pro zařízení SD:

  - INIT „path“ to change current folder

  - DIR „SD:“ to list directory, it is necessary to repeat the command for folders with more files in it

- Important commands are defined on F4 and F5 keys

- We recommend to insert /MZR/rambas.mzr to RAM: device before the launch of the BASIC.

OPENINGB000.MZF

- MZ-700 BASIC demo, it runs from BASIC_Unicard, cannot be run from Unicard Manager.

- Enter the INIT „/MZF“ and DIR „SD:“ commands to choose folder /MZF as default and list its conents.

- Enter RUN „OPENINGB000.MZF“

  - TIP: Press F1 on the beginning of the line with BTX „OPENING 800“ list.

Hu-Basic V2.0a.mzt

- Hu-BASIC 2.0A for SHARP MZ-700.

- It can work with the emulated EMM: disk (use FILES “EMM:” command).

Cosmic Cruiser 3.mzf

- Famous SHARP MZ-1500 game. Can be run directly from Unicard Manager.

/MZQ 			; Quick Disk images

mzfloader.mzq

- Unicard Manager loader.

- RESET or RESET+Q runs the Unicard Manager from mapped emulated QD.

QDisk-5Z001_74.mzq

- Common SHARP MZ-1500 BASIC 5Z-001 with support for emulated QD: on port 0x74 and RAM: disk.

- We recommend to insert /MZQ/qdtestbas.mzq file to QD: and /MZR/rambas.mzr to RAM: device before the launch of the BASIC.

Flappy_A.mzq
Flappy_B.mzq

- Two parts image of famous Flappy game for MZ-1500.

- Requires changed MZ-1500 ROM monitor in 2764 EPROM with content in /EPROM/9Z-502M_eprom74.bin

- Insert _A.mzq in QD:, reset computer, wait few moments (the Unicard will auto swap _A for _B) and press CR. The game will load the second part and start.

/MZR			; RAM disk images

EMM1.mze

- EMM: disk 640 kB image, it works with Hu-BASIC

ramdisc.mzr

- RAM: disk image, that contains pro Unicard Manager loader. BASIC can destroy it.

rambas.mzr

- Sampler RAM: disk content for BASIC MZ-1Z016A.

- When you run BASIC, the contents is always saved to the SD card and is persistent. You can SAVE, LOAD, DELETE and use even more BASIC file operations on it.

madonna2.mzr

- RAM: disk content required for /MZF/madonna2.mzf video demo.

/DSK			; floppy disk images

MZ-5Z001.DSK

- BASIC 5Z-001 converted to floppy disk image. Do you know any MZ-1500 software on Floppy Disk?

/SYNCVGA		; small programs for simple VGA resolution change

VGA_640x400@70.mzf
VGA_640x400@85.mzf
VGA_640x480@60.mzf
VGA_640x480@73.mzf
VGA_640x480@75.mzf
VGA_640x480@85.mzf
VGA_640x480@100.mzf
VGA_720x400@85.mzf
VGA_768x576@60.mzf
VGA_768x576@72.mzf
VGA_768x576@75.mzf
VGA_768x576@85.mzf
VGA_800x600@56.mzf
VGA_800x600@60.mzf
VGA_800x600@72.mzf
VGA_800x600@75.mzf

- The output VGA resolution of the Unicard is changed after the program is started.

- The resolution change is valid until the computer or Unicard is switched off.

- If you want to change the resolution permanently, use the unicard.cfg file.

FAQ

After a reset, the Unicard Manager won't start. What should I do?

- There are several possibilities, for example, that you have a mapped floppy drive from which the system boots first.

- Press the Q key, hold it, and press the RESET button on the back, hold the Q key until the computer boots from the emulated QD: device.

- If the previous step doesn't help because you don't have a loader in QD: or there is nothing there, press the M key, hold it, and press the RESET key at the back, hold the M key until the computer boots into the ROM Monitor. Then enter the EB command and press the CR key. The computer should boot from the emulated spoofed ROM/SRAM disk, which by default contains the loader.

- If none of the previous steps help, put the SD card from the Unicard into the PC and delete and re-copy the contents of the unicard, MZR and MZQ directories to the SD card from the sample contents archive.

The EB command in my monitor says CHECK SUM ER. What should I do?

- You have corrupted the of the RAM: disk contents, probably after running BASIC or CP/M

- Put the SD card from the Unicard into the PC and delete and re-copy the contents of the unicard, MZR and MZQ directories to the SD card from the sample contents archive.

Support

Report bugs to martin@8bity.cz. Please be patient. Thank you! 😉