SHARP MZ-800 Unicard demo disk quick guide

The sample disk contains a sample configuration and several SHARP MZ programs to get you acquainted with the Unicard. The basic program to control the Unicard is called Unicard Manager. It starts automatically. You can use it to directly run MZF/MZT files (BASIC MZF/MZT files must be run from BASIC), map floppy drives, Quick Disks, RAM disks, etc. There are several ways to start the manager:

- If /unicard/ramdisc.cfg file exists and points to /MZR/ramdisc.mzr in which the loader is.

- If the contents of the RAM disk are unbootable (e.g. /MZR/rambas.mzr), Unicard will replace it with the loader after MZ reset for a while, until the MZ gets it and then returns the desired contents. The SHARP MZ will thus boot automatically from the spoofed loader. It is also possible to jump to the monitor via RESET+M and start the loader via the EB command. (This is not possible if the RAM disk is marked as bootable but has a CHECK SUM ERROR, e.g. after running BASIC with the bootable RAM disk mapped. In this case, you must reload the corresponding MZR file from the original archive.)

- If there is a /unicard/qd.cfg file pointing to /MZQ/mzfloader.mzq, which contains the loader.

- If the previous file exists but the RAM disk is mapped with faulty contents, it is possible to force the Quick Disk to start via RESET+Q or RESET+M and entering the QL command followed by a double CR.

SD card contents

/ (root)

unicard.app

- The latest version of the Unicard Mk3b firmware for SHARP MZ-800.

- Ensures automatic reprogramming of the Unicard firmware on the first start-up.

- You can delete it from the SD card after programming or leave it as it is, it doesn't matter, the Unicard firmware is reprogrammed only once.

Unicard_MZ-800_SD-sample.rtf

- This quick guide. You can remove it from the SD card, Unicard does not use it ;-)

/unicard 		; Unicard configuration folder

emm.cfg

- Contains the path to the mapped EMM: device (EMM RAM disk, default is /MZR/EMM1.mze)

fd0.cfg, fd1.cfg, fd2.cfg a fd3.cfg

- Contains the path to the mapped floppy disk drive A:, B:, C: a D:

mgr800.mzf

- Unicard Manager – the program that allows you to directly run MZF/MZT files and to configure emulated peripherals mapping.

mouse.cfg

- Presence of the file enables PS/2 mouse emulation on port 71h.

- Programs with mouse support use the second built-in JoyPort on F1h, but it would overlap with Unicard, so you must use modified programs, e.g. Lemmings from the /DSK directory.

mzfloader.cfg

- Points to the Unicard Manager MZF executable („/unicard/mgr800.mzf“ by default).

- It is required for integrated spoofed SRAM disk or loaders in QD: mzfloader.mzq or RAM: ramdisc.mzr, as the loader need to know where wo get the Unicard Manager to run.

network.cfg

- Allows you to set the MAC address, IP address, mask and gateway for an integrated Web or FTP server
default IP: 192.168.1.111
default MASK: 255.255.255.0
default GW: 192.168.1.1

- Also includes a username and password to log in to the integrated FTP server.
default user: unicard
default pass: unic1234

qd.cfg

- Points to the currently mapped QD image: (default loader in MZQ/mzfloader.mzq)

- If the file does not exist, the QD: device is not emulated.

ramdisc.cfg

- Points to the currently mapped RAM: disk (default loader in MZR/ramdisc.mzr).

/unicard/_not_used/ushelldisabled.cfg

- Disables UniUSARTShell on the USB port and makes the USB available for serial communication on the emulated serial card (RS2:).

- If you move the file one directory up to the /unicard folder, the UniUSARTShell will not be accessible on the USB port for commands and file transfer, but it will be accessible on RS2: channel of the emulated MZ-1E24 serial card. The RS1: channel is always mapped to the physical serial port (DSUB-9 connector on the Unicard back panel).

ATTENTION! If you put files ramdisc.dat or mzfloader.mzq to the /unicard folder, they will be "hard" mapped to RAM: or QD: regardless of any other configuration files and settings in Unicard Manager.

/MZF (programs for easy launching directly from Unicard Manager or loading in Unicard BASIC)

AUTO RUN.mzf

- AUTO RUN file for BASIC_5Z009_for_unicard.mzf

- Automatically starts when the BASIC starts and makes DIR "SD:"

- Cannot be run from the Unicard Manager, it is a BASIC program

BASIC_1Z-06A.mzf

- Common SHARP MZ-800 BASIC with support for QD: and RAM: disk.

- We recommend to insert /MZQ/qdtestbas.mzq file to QD: and /MZR/rambas.mzr to RAM: device before the launch of the BASIC.

BASIC_5Z009_for_unicard.mzf

- BASIC with Unicard support (“SD:” device), support for “QD:” device was removed.

- Extended command are INIT, DIR, SAVE, LOAD pro zařízení SD:

  - INIT „path“ to change current folder

  - DIR „SD:“ to list directory, it is necessary to repeat the command for folders with more files in it

- Important command are defined on F4 and F5 keys

- We recommend to insert /MZR/rambas.mzr to RAM: device before the launch of the BASIC.

OPENING DATA.mzf 
OPENING 800.mzf

- BASIC demo, it runs from BASIC_Unicard, cannot be run from Unicard Manager.

- OPENING 800 will load the OPENING DATA after the RUN.

- Enter the INIT „/MZF“ and DIR „SD:“ commands to choose folder /MZF as default and list its conents.

- Enter RUN „OPENING 800“

  - TIP: Press F1 on the beginning of the line with BTX „OPENING 800“ list.

MZTSTRS1.BAS.mzf
MZTSTRS2.BAS.mzf

- Simple RS-232c test program written in BASIC (RS1: a RS2:)

- It uses standard BASIC command to work with the emulated MZ-1E24 interface.

- It runs from BASIC_Unicard, cannot be run from Unicard Manager.

TELNET.BAS.mzf

- Simple TELNET client written in BASIC.

- It runs from BASIC_Unicard, cannot be run from Unicard Manager.

Hu-Basic V2.0a.mzt

- Hu-BASIC 2.0A for SHARP MZ-700.

- It can work with the emulated EMM: disk (use FILES “EMM:” command).

Flappy.mzf
Galao.mzf
Zexas.mzf

- Famous SHARP MZ-800 games. Can be run directly from Unicard Manager.

- ZEXAS game requires SW4 switch up (MZ-700 mode).

Fxs4.mzf

- Famous music demo. Can be run directy from Unicard Manager.

mzmessexe.mzf

- MZ messenger chat client, requires working TCP/IP and MZ Chat server, which does not have to always run (ask Martin on martin@8bity.cz if the server is not online).

- You can find Windows version of the MZ chat client in /PC folder (.NET Framework is required).

Y2K.mzf

- A simple but technically interesting game for the MZ-700, that uses different characters in each micro-line to create fine graphics even on the MZ-700's character mode. It requires a Japanese character generator, so confirm the start not with "Y" but with "J".

madonna2.mzf

- Famous video demo. File /MZR/madonna2.mzr is required as RAM disk content.

Rdtest23.mzf

- RAM disk test for MZ-800.

- ATTENTION! It destroys RAM disk content.

Sharp_Test.mzf

- Test program for SHARP MZ-800.

/MZQ 			; Quick Disk images

mzfloader.mzq

- Unicard Manager loader.

- Contains BASIC MZ-800 demo program.

- RESET or RESET+Q runs the Unicard Mana from mapped emulated QD.

qdtestbas.mzq

- Empty QD: disk with simple AUTO RUN for BASIC MZ-1Z016A.

/MZR			; RAM disk images

EMM1.mze

- EMM: disk 640 kB image, it works with Hu-BASIC

ramdisc.mzr

- RAM: disk image, that contains pro Unicard Manager loader. BASIC can destroy it.

rambas.mzr

- Sampler RAM: disk content for BASIC MZ-1Z016A.

- When you run BASIC, the contents is always saved to the SD card and is persistent. You can SAVE, LOAD, DELETE and use even more BASIC file operations on it.

madonna2.mzr

- RAM: disk content required for /MZF/madonna2.mzf video demo.

/DSK			; floppy disk images

BASIC_2Z046_with_DEMO.DSK

- Contains Disk BASIC MZ-2Z046 with demo program.

- You can run the DEMO with RUN“OPENING 800“ command. Just overwrite BTX with RUN 2 lines above the cursor and press CR.

CPM_LEMMINGS_UNICARD.dsk

- Famous Lemmings game updated with Unicard emulated PS/2 mouse support.

- It requires file mouse.cfg in the /unicard/ folder.

CPMv41 Prg CPM.dsk

- CP/M boot disk with UNIUTIL program for Unicard manipulation.

- We recommend to map this disk as drive A:

Martin144.dsk

- Floppy disk image 1,44 MB with UNIUTIL source codes, Turbo Pascal 3.0 and NC.

- We recommend to map this disk as drive D: and boot from A: [CPMv41 Prg CPM.dsk]

/SYNCVGA		; small programs for simple VGA resolution change

VGA_640x400@70.mzf
VGA_640x400@85.mzf
VGA_640x480@60.mzf
VGA_640x480@73.mzf
VGA_640x480@75.mzf
VGA_640x480@85.mzf
VGA_640x480@100.mzf
VGA_720x400@85.mzf
VGA_768x576@60.mzf
VGA_768x576@72.mzf
VGA_768x576@75.mzf
VGA_768x576@85.mzf
VGA_800x600@56.mzf
VGA_800x600@60.mzf
VGA_800x600@72.mzf
VGA_800x600@75.mzf

- The output VGA resolution of the Unicard is changed after the program is started.

- The resolution change is valid until the computer or Unicard is switched off.

- If you want to change the resolution permanently, use the unicard.cfg file.

FAQ

The manager has partial red stripes across it, the graphics of some programs look weird.

Unicard Manager and some other programs require extended video RAM to run. Install two integrated DRAM circuits 4416 or 4461 (41416, 41464) in your MZ-800. The IC sockets are located under the keyboard on the lower right corner.

After a reset, the Unicard Manager won't start. What should I do?

- There are several possibilities, for example, that you have a mapped floppy drive from which the system boots first.

- Press the Q key, hold it, and press the RESET button on the back, hold the Q key until the computer boots from the emulated QD: device.

- If the previous step doesn't help because you don't have a loader in QD: or there is nothing there, press the M key, hold it, and press the RESET key at the back, hold the M key until the computer boots into the ROM Monitor. Then enter the EB command and press the CR key. The computer should boot from the emulated spoofed ROM/SRAM disk, which by default contains the loader.

- If none of the previous steps help, put the SD card from the Unicard into the PC and delete and re-copy the contents of the unicard, MZR and MZQ directories to the SD card from the sample contents archive.

The EB command in my monitor says CHECK SUM ER. What should I do?

- You have corrupted the of the RAM: disk contents, probably after running BASIC or CP/M

- Put the SD card from the Unicard into the PC and delete and re-copy the contents of the unicard, MZR and MZQ directories to the SD card from the sample contents archive.

Support

Report bugs to martin@8bity.cz. Please be patient. Thank you! 😉